AUTOID Swatch-2.0M4-9.0

PRODUCT SPECIFICATION V1.0

JIANGSU SEUIC TECHNOLOGY CO., LTD

# AUTOID Swatch-2.0M4-9.0 Performance, Physical and Environmental Parameters

| - CPU | - MTK 4-core 2.0GHz |
| --- | --- |
| - Operating system | - Android 9.0 |
| - RAM | - LPDDR3 3GB |
| - ROM | - 32GB |
| - Interface | - TYPE-C |
| - Keys | - 2 keys (power supply, custom), 1 million times life |
| - Screen | - 2.8 inch full view LCD, capacitive 3-point touch. |
|  | - The resolution is 640 × 480 and the maximum brightness is 350nit |
| - Power Supply | - Non detachable 3300mah lithium ion rechargeable battery, 500 cycles, capacity more than 80% |
| - Audio frequency | - Built in speaker, 8 Ω, 1 W * 1. Built in dual microphone |
| - Sensor | - Acceleration sensor, proximity light sensor, electronic compass, gyroscope, barometer. Support to raise your hand to light up the screen. |
| - Vibrating motor | - support |
| - Size | - 54.7*80*17mm |
| - Weight (including standard battery) | - 100 g No strap |
| - Working temperature | - -20℃ to +60℃ |
| - Storage temperature | - -30℃ to +70℃ |
| - Humidity | - 5% to 95% RH No condensation state |
| - Waterproof and dustproof industrial grade | - IP67 |
| - Drop Level | - 1.2m Marble Falls, 5 rounds on 6 sides, 30 times |
| - Electrostatic discharge (ESD) | - Air discharge of ± 15kV and direct discharge of ± 8Kv |

# AUTOID Swatch-2.0M4-9.0 Data Acquisition Function Parameters

| - Function 1 | - Geographic Location Information Collection |
| --- | --- |
| Location | - GPS + GLONASS + Beidou + galileeo, support AGPs |
| Working mode | - GPS main positioning, Beidou, GLONASS, galileeo auxiliary positioning |
| Frequency | - GPS:L1(1.575GHz)；Beidou : B1(1.561GHz)； GLONASS:L1(1.602GHZ) |
| Time positioning | Cold start ≤ 40s, hot start ≤ 3S |
| Accuracy | - 5-10m，open sky |
| - Function 2 | - Rear Camera Image Acquisition |
| Pixel | 1300 ten thousand |
| Photo taking function | PDAF, f = 2.0 |
| Flash lamp | Flash, 1A flash, torch mode |

# AUTOID Swatch-2.0M4-9.0 Function Parameters of Wireless Data Communication

| - Function 1 | - WIFI |
| --- | --- |
| Agreement | IEEE 802.11a/b/g/n/ac（2.4G/5G Dual frequency WIFI） |
| Frequency range | Depending on the country (region), 2.4GHz is usually 2.412ghz-2.472ghz; 5GHz is usually 5.170ghz-5.825ghz |
| Working channel | CH1~CH13，CH34-CH140，CH149~CH165 |
| Security | OPEN、WEP、WPA2-PSK（TKIP+AES）、TLS、TTLS (MS-CHAP),、TTLS(MS-CHAP v2)、TTLS (CHAP)、TTLSMD5、TTLS-PAP、PEAP-TLS、PEAP(MS-CHAP v2)、AES etc. |
| Transmission distance | 2.4G greater than 50m, 5g greater than 40m (open area) |
| Data rate | 802.11a: 6,9,12,18,24,36,48,54Mbps 802.11b: 1, 2,5.5,11Mbps 802.11g: 6,9,12,18,24,36,48,54Mbps 802.11n: 6.5,13,19.5,26,39,52,58.5,65Mbps（20MHz,Long GI,800ns) 802.11n: 7.2,14.4,21.7,28.9,43.3,57.8,65,72.2Mbps（20MHz,short GI,400ns) |
| - Function 2 | - Wireless WAN |
| Frequency band | GSM：850/900/1800/1900 TD-SCDMA：B34/B39 WCDMA：B1/B2/B5/B8 CDMA：BC0 TDD-LTE：B34/B38/B39/B40/B41 FDD-LTE：B1/B3/B5/B7/B8 Support VoLTE |
| - Function 3 | - Wireless personal area network Bluetooth |
| Standard | - Bluetooth 5.0 (dual mode, ble supported) |
| Distance | - More than 20 meters |

- AUTOID Swatch-2.0M4-9.0 THE THIRD PARTY APPLICATION DEVELOPMENT SUPPORT

| System Software | - Android 9.0 |
| --- | --- |
| System programming environment | - Android Studio |

- AUTOID Swatch-2.0M4-9.0 PERIPHERALS AND ACCESSORIES

| Standard Accessories | - Charger ×1, TYPE-C Cable×1,Watch strap ×1,Steel film x1 |
| --- | --- |